Accord Specification 2009

| Engineering |  |  |  |
| --- | --- | --- | --- |
|  | LX / LX-P | EX / EX-L | EX V-6 / EX-L V-6 |
| Engine Type | In-Line 4-Cylinder | In-Line 4-Cylinder | V-6 |
| Engine Block/Cylinder Head | Aluminum-Alloy | Aluminum-Alloy | Aluminum-Alloy |
| Displacement (cc) | 2354 | 2354 | 3471 |
| Horsepower @ rpm (SAE net) | 177 @ 6500 | 190 @ 7000 | 271 @ 6200 |
| Torque (lb-ft @ rpm) | 161 @ 4300 | 162 @ 4400 | 254 @ 5000 |
| Redline | 6800 | 7100 | 6800 |
| Bore and Stroke (mm) | 87 x 99 | 87 x 99 | 89 x 93 |
| Compression Ratio | 10.5 : 1 | 10.5 : 1 | 10.5 : 1 |
| Valve Train | 16-Valve DOHC i-VTEC® | 16-Valve DOHC i-VTEC® | 24-Valve SOHC i-VTEC® |
| Multi-Point Fuel Injection | Standard | Standard | Standard |
| Drive-by-Wire™ Throttle System | Standard | Standard | Standard |
| Variable Cylinder Management™ (VCM®) |  |  | Standard |
| Active Noise Cancellation™ (ANC) |  | Standard | Standard |
| CARB Emissions Rating | ULEV-2/PZEV* | ULEV-2/PZEV* | ULEV-2/PZEV* |
| Direct Ignition System with Immobilizer | Standard | Standard | Standard |
| 100K +/- Miles No Scheduled Tune-Ups† | Standard | Standard | Standard |
| Transmissions |  |  |  |
|  | LX / LX-P | EX / EX-L | EX V-6 / EX-L V-6 |
| 5-Speed Manual Transmission | Standard | Standard |  |
| Gear Ratios: 1st: 3.267, 2nd: 1.778, 3rd: 1.154, 4th: 0.870, 5th: 0.647, Reverse: 3.583, Final Drive Ratio: 4.39 |  |  |  |
| 5-Speed Automatic Transmission (available) | Standard | Standard |  |
| Gear Ratios: 1st: 2.652, 2nd: 1.517 (LX/LX-P), 1.614 (EX/EX-L), 3rd: 1.037 (LX/LX-P), 1.082 ( EX/EX-L), 4th: 0.738 (LX/LX-P), 0.773 (EX/EX-L), 5th: 0.566, Reverse: 2.000, Final Drive Ratio: 4.44 |  |  |  |
| 5-Speed Automatic Transmission |  |  | Standard |
| Gear Ratios: 1st: 2.697, 2nd: 1.606, 3rd: 1.071, 4th: 0.766, 5th: 0.612, Reverse: 1.889, Final Drive Ratio: 4.31 |  |  |  |
| Body/Suspension/Chassis |  |  |  |
|  | LX / LX-P | EX / EX-L | EX V-6 / EX-L V-6 |
| Unit-Body Construction | Standard | Standard | Standard |
| Double Wishbone Front Suspension | Standard | Standard | Standard |
| Independent Multi-Link Rear Suspension | Standard | Standard | Standard |
| Stabilizer Bar (mm, front/rear) | 26.5 / 13.0 | 26.5 / 13.0 | 26.5 / 14.0 |
| Front Shock Tower Bar |  | Standard | Standard |
| Variable Gear Ratio (VGR) Power-Assisted Rack-and-Pinion Steering | Standard | Standard | Standard |
| Steering Wheel Turns, Lock-to-Lock | 2.56 | 2.56 | 2.56 |
| Steering Ratio | 13.08 | 13.08 | 13.08 |
| Turning Diameter, Curb-to-Curb (ft) | 37.7 | 37.7 | 37.7 |
| Power-Assisted Ventilated Front Disc/Solid Rear Disc Brakes (in) | 11.1 / 11.1 | 11.8 / 11.1 | 11.8 / 11.1 |
| Wheels | 16" with Full Covers (LX) 16" Alloy (LX-P) | 17" Alloy | 17" Alloy |
| All-Season Tires | P215/60 R16 94H | P225/50 R17 93V | P225/50 R17 93V |
| Exterior Measurements |  |  |  |
|  | LX / LX-P | EX / EX-L | EX V-6 / EX-L V-6 |
| Wheelbase (in) | 110.2 | 110.2 | 110.2 |
| Length (in) | 194.1 | 194.1 | 194.3 |
| Height (in) | 58.1 | 58.1 | 58.1 |
| Width (in) | 72.7 | 72.7 | 72.7 |
| Track (in, front/rear) | 62.6 / 62.6 | 62.2 / 62.2 | 62.2 / 62.2 |
| Curb Weight (lbs, MT/AT) | 3230 / 3289 (LX) 3236 / 3298 (LX-P) | 3349 / 3408 (EX) 3373 / 3433 (EX-L) | NA / 3567 (EX-V-6) NA / 3600 (EX-L V-6) |
| Interior Measurements |  |  |  |
|  | LX / LX-P | EX / EX-L | EX V-6 / EX-L V-6 |
| Headroom (in, front/rear) | 41.4 / 38.5 |  |  |
| Headroom with Moonroof (in, front/rear) |  | 39.0 / 37.2 | 39.0 / 37.2 |
| Legroom (in, front/rear) | 42.5 / 37.2 | 42.5 / 37.2 | 42.5 / 37.2 |
| Shoulder Room (in, front/rear) | 58.2 / 56.4 | 58.2 / 56.4 | 58.2 / 56.4 |
| Hiproom (in, front/rear) | 56.6 / 54.3 | 56.6 / 54.3 | 56.6 / 54.3 |
| Cargo Volume (cu ft) | 14 | 14 | 14 |
| Passenger Volume (cu ft) | 106 | 101 | 101 |
| Seating Capacity | 5 | 5 | 5 |
| EPA Mileage Estimates**/Capacities |  |  |  |
|  | LX / LX-P | EX / EX-L | EX V-6 / EX-L V-6 |
| 5-Speed Manual (City/Highway/Combined) | 22 / 31 / 25 | 22 / 31 / 25 |  |
| 5-Speed Automatic (City/Highway/Combined) | 21 / 30 / 24 | 21 / 30 / 24 | 19 / 29 / 22 |
| Crankcase (qt) | 5.6 | 5.6 | 5.3 |
| Coolant System (qt, MT/AT) | 8.6 / 8.5 | 8.6 / 8.5 | NA / 9.1 |
| Fuel (gal) | 18.5 | 18.5 | 18.5 |
| Required Fuel | Regular Unleaded | Regular Unleaded | Regular Unleaded |
| Safety |  |  |  |
|  | LX / LX-P | EX / EX-L | EX V-6 / EX-L V-6 |
| 3-Point Seat Belts at all Seating Positions | Standard | Standard | Standard |
| Front 3-Point Seat Belts with Automatic Tensioning System | Standard | Standard | Standard |
| Adjustable Front Seat Belt Anchors | Standard | Standard | Standard |
| Driver's and Front Passenger's Seat Belt Reminder | Standard | Standard | Standard |
| Dual-Stage, Dual-Threshold Front Airbags (SRS) | Standard | Standard | Standard |
| Dual-Chamber Front Side Airbags with Passenger-Side Occupant Position Detection System (OPDS) | Standard | Standard | Standard |
| Side Curtain Airbags | Standard | Standard | Standard |
| Driver's and Front Passenger's Active Head Restraints | Standard | Standard | Standard |
| Advanced Compatibility EngineeringTM (ACETM) Body Structure | Standard | Standard | Standard |
| Vehicle Stability Assist™ (VSA®) with Traction Control | Standard | Standard | Standard |
| Anti-Lock Braking System (ABS) | Standard | Standard | Standard |
| Electronic Brake Distribution (EBD) | Standard | Standard | Standard |
| Brake Assist | Standard | Standard | Standard |
| Tire Pressure Monitoring System (TPMS) | Standard | Standard | Standard |
| Daytime Running Lights (DRL) | Standard | Standard | Standard |
| Side-Impact Door Beams | Standard | Standard | Standard |
| Outboard Lower Anchors and Tethers for CHildren (LATCH) | Standard | Standard | Standard |
| Child-Seat Tether Anchor (Rear-Center) | Standard | Standard | Standard |
| Child-Proof Rear Door Locks | Standard | Standard | Standard |
| Immobilizer Theft-Deterrent System | Standard | Standard | Standard |
| Emergency Trunk Release | Standard | Standard | Standard |
| Exterior Features |  |  |  |
|  | LX / LX-P | EX / EX-L | EX V-6 / EX-L V-6 |
| One-Touch Power Moonroof with Tilt Feature |  | Standard | Standard |
| Integrated Rear Window Antenna | Standard | Standard | Standard |
| Alloy Wheels | 16" (LX-P) | 17" | 17" |
| Security System | LX-P | Standard | Standard |
| Remote Entry System with Power Window Control | Standard | Standard | Standard |
| Variable Intermittent Windshield Wipers | Standard | Standard | Standard |
| Multi-Reflector Halogen Headlights with Auto-off | Standard | with Auto-On/Off (EX-L) | with Auto-On/Off (EX-L V-6) |
| Hood Struts |  |  | Standard |
| Fog Lights |  |  | Standard |
| Body-Colored Door Handles | Standard | Standard | Chrome |
| Body-Colored Power Side Mirrors | Standard | Heated | Heated |
| Body-Colored Impact-Absorbing Bumpers | Standard | Standard | Standard |
| Chrome Exhaust Finisher | LX-P | Standard | Dual Exhaust |
| Chrome Window Trim | Standard | Standard | Standard |
| Comfort & Convenience |  |  |  |
|  | LX / LX-P | EX / EX-L | EX V-6 / EX-L V-6 |
| Air Conditioning with Air-Filtration System | Standard | EX | EX V-6 |
| Dual-Zone Automatic Climate Control System |  | EX-L | EX-L V-6 |
| Honda Satellite-Linked Navigation System™ with Voice Recognition |  | Available (EX-L) | Available (EX-L V-6) |
| Bluetooth® HandsFreeLink®†† |  | With Navigation | With Navigation |
| Interface Dial with Scrolling and Multi-Line Display |  | With Navigation | With Navigation |
| Selector Knob with Scrolling and Multi-Line Display |  | Standard | Standard |
| Power Windows with Auto-Up/Down Driver's and Front Passenger's Windows | LX-P | Standard | Standard |
| Illuminated Power Window Switches | LX-P | Standard | Standard |
| Power Door Locks/Programmable Auto-Locking Doors | Standard | Standard | Standard |
| Cruise Control | Standard | Standard | Standard |
| Leather-Wrapped Steering Wheel |  | EX-L | EX-L V-6 |
| Leather-Wrapped Shift Knob |  | EX-L 5MT |  |
| Illuminated Steering Wheel-Mounted Audio and Cruise Controls | Standard | Standard | Standard |
| Steering Wheel-Mounted Navigation and Phone Controls |  | With Navigation | With Navigation |
| Tilt and Telescopic Steering Column | Standard | Standard | Standard |
| HomeLink® Remote System*** |  |  | EX-L V-6 |
| Premium Interior Accents |  | Standard | Standard |
| Center Console with Sliding Armrest and Storage Compartment | Standard | Standard | Standard |
| Beverage Holders, Front and Rear | Standard | Standard | Standard |
| Driver's and Front Passenger's Illuminated Vanity Mirrors | Standard | Standard | Standard |
| Automatic Dimming Rearview Mirror |  | EX-L | EX-L V-6 |
| Blue Ambient Console Lighting |  | Standard | Standard |
| Map Lights | Standard | Standard | Standard |
| Sunglasses Holder | Standard | Standard | Standard |
| Coin Box | Standard | Standard | Standard |
| 12-Volt Power Outlets | Standard | Standard | Standard |
| Lockable Glove Compartment with Light | Standard | Standard | Standard |
| Driver- and Passenger-Side Seatback Pockets | Standard | Standard | Standard |
| Remote Fuel Filler Door Release | Standard | Standard | Standard |
| Remote Trunk Release with Lock | Standard | Standard | Standard |
| Electronic Remote Trunk Release | Standard | Standard | Standard |
| Rear Window Defroster | Standard | Standard | Standard |
| Cargo Area Light | Standard | Standard | Standard |
| Floor Mats | Standard | Standard | Standard |
| Side Door Pockets | Standard | Standard | Standard |
| Seating |  |  |  |
|  | LX / LX-P | EX / EX-L | EX V-6 / EX-L V-6 |
| Leather-Trimmed Seats |  | EX-L | EX-L V-6 |
| Heated Front Seats |  | EX-L | EX-L V-6 |
| Driver's Seat with 8-Way Power Adjustment | LX-P | Standard | Standard |
| Driver's Seat with Power Lumbar Support |  | Standard | Standard |
| Driver's Seat with Manual Height Adjustment | LX |  |  |
| Passenger's Seat with 4-Way Power Adjustment |  |  | EX-L V-6 |
| Fold-Down Rear Seatback | Standard | Standard | Standard |
| Fold-Down Rear-Seat Center Armrest/Trunk Pass-Through | Standard | Standard | Standard |
| Audio Systems |  |  |  |
|  | LX / LX-P | EX / EX-L | EX V-6 / EX-L V-6 |
| 160-Watt AM/FM/CD Audio System with 6 Speakers | Standard |  |  |
| 160-Watt AM/FM/6-Disc In-Dash Audio System with 6 Speakers |  | EX | EX V-6 |
| 270-Watt AM/FM/6-Disc In-Dash Premium Audio System with 7 Speakers including Subwoofer |  | EX-L | EX-L V-6 |
| Radio Data System (RDS) | Standard | Standard | Standard |
| MP3/Windows Media® Audio (WMA) Playback Capability | Standard | Standard | Standard |
| MP3/Auxiliary Input Jack | Standard | Standard | Standard |
| XM® Radio |  | EX-L | EX-L V-6 |
| Speed-Sensitive Volume Control (SVC) | Standard | Standard | Standard |
| Instrumentation |  |  |  |
|  | LX / LX-P | EX / EX-L | EX V-6 / EX-L V-6 |
| Backlit Gauges | Standard | Standard | Standard |
| Tachometer | Standard | Standard | Standard |
| Digital Odometer and Digital Trip Meters (2) | Standard | Standard | Standard |
| Maintenance Minder™ System | Standard | Standard | Standard |
| Compass |  | EX-L without Navigation | EX-L V-6 without Navigation |
| Door-/Trunk-Open Indicator | Standard | Standard | Standard |
| Fuel and Coolant Temperature Indicators | Standard | Standard | Standard |
| Low-Oil Pressure and Brake Fluid Indicators | Standard | Standard | Standard |
| Exterior Temperature Indicator |  | EX-L | EX-L V-6 |
| Vehicle Stability Assist (VSA®) Indicator | Standard | Standard | Standard |
| Passenger-Side Side Airbag-Off Indicator | Standard | Standard | Standard |

* AT models are PZEV rated in California and states that have adopted California emission standards. MT models are ULEV-2 rated in all 50 states.

† Does not apply to fluid and filter changes. Will vary with driving conditions. See your owner's manual for more details.

** Based on 2009 EPA mileage estimates, reflecting new EPA fuel economy methods beginning with 2008 models. Use for comparison purposes only. Do not compare to models before 2008. Your actual mileage will vary depending on how you drive and maintain your vehicle.

†† The Bluetooth® word mark and logos are owned by the Bluetooth SIG, Inc., and any use of such marks by Honda Motor Co., Ltd., is under license.

*** HomeLink® is a registered trademark of Prince Corporation, a wholly owned subsidiary of Johnson Controls™.