Civic Coupe Specification 2009

| Engineering |  |  |  |
| --- | --- | --- | --- |
|  | DX | LX | EX/EX-L |
| Engine Type | In-Line 4-Cylinder | In-Line 4-Cylinder | In-Line 4-Cylinder |
| Engine Block/Cylinder Head | Aluminum-Alloy | Aluminum-Alloy | Aluminum-Alloy |
| Displacement (cc) | 1799 | 1799 | 1799 |
| Horsepower @ rpm (SAE net) | 140 @ 6300 | 140 @ 6300 | 140 @ 6300 |
| Torque (lb-ft @ rpm) | 128 @ 4300 | 128 @ 4300 | 128 @ 4300 |
| Redline (rpm) | 6800 | 6800 | 6800 |
| Bore and Stroke (mm) | 81 x 87.3 | 81 x 87.3 | 81 x 87.3 |
| Compression Ratio | 10.5:1 | 10.5:1 | 10.5:1 |
| Valve Train | 16-Valve SOHC i-VTEC® | 16-Valve SOHC i-VTEC® | 16-Valve SOHC i-VTEC® |
| Multi-Point Fuel Injection | Standard | Standard | Standard |
| Drive-by-Wire™ Throttle System | Standard | Standard | Standard |
| Front-Wheel Drive | Standard | Standard | Standard |
| CARB Emissions Rating* | ULEV-2 | ULEV-2 | ULEV-2 |
| Direct Ignition System with Immobilizer | Standard | Standard | Standard |
| 100K +/- Miles No Scheduled Tune-Ups† | Standard | Standard | Standard |
| Transmissions |  |  |  |
|  | DX | LX | EX/EX-L |
| 5-Speed Manual Transmission | Standard | Standard | Standard |
| Gear Ratios: 1st: 3.143, 2nd: 1.870, 3rd: 1.235, 4th: 0.949, 5th: 0.727, Reverse: 3.308, Final Drive: 4.29 |  |  |  |
| Compact 5-Speed Automatic Transmission (available) | Standard | Standard | Standard |
| Gear Ratios: 1st: 2.666, 2nd: 1.534, 3rd: 1.022, 4th: 0.721, 5th: 0.525, Reverse: 1.957, Final Drive: 4.44 |  |  |  |
| Gear Ratios: 1st: 3.267, 2nd: 2.130, 3rd: 1.517, 4th: 1.147, 5th: 0.921, 6th: 0.659, Reverse: 3.583, Final Drive: 4.76 |  |  |  |
| Body/Suspension/Chassis |  |  |  |
|  | DX | LX | EX/EX-L |
| Unit-Body Construction | Standard | Standard | Standard |
| MacPherson Strut Front Suspension | Standard | Standard | Standard |
| Multi-Link Rear Suspension | Standard | Standard | Standard |
| Stabilizer Bar (mm., front/rear) | 25.4 / NA | 25.4 / 11.0 | 25.4 / 11.0 |
| Power-Assisted Rack-and-Pinion Steering | Standard | Standard | Standard |
| Steering Wheel Turns, Lock-to-Lock | 2.73 | 2.67 | 2.67 |
| Steering Ratio | 13.62 | 13.73 | 13.73 |
| Turning Diameter, Curb-to-Curb (ft.) | 35.4 | 35.4 | 35.4 |
| Power-Assisted Ventilated Front Disc/Rear Drum Brakes (in., front/rear) | 10.3 / 7.9 | 10.3 / 7.9 |  |
| Power-Assisted Ventilated Front Disc/Solid Rear Disc Brakes |  |  | 10.3 / 10.2 |
| Wheels | 15" with Full Covers | 16" with Full Covers | 16" Alloys |
| All-Season Tires | P195 / 65 R15 89H | P205 / 55 R16 89H | P205 / 55 R16 89H |
| Compact Spare Tire | Standard | Standard | Standard |
| Exterior Measurements |  |  |  |
|  | DX | LX | EX/EX-L |
| Wheelbase (in.) | 104.3 | 104.3 | 104.3 |
| Length (in.) | 175.5 | 175.5 | 175.5 |
| Height (in.) | 55.0 | 55.0 | 55.0 |
| Width (in.) | 68.9 | 68.9 | 68.9 |
| Track (in., front/rear) | 59.0/60.1 | 59.0/60.1 | 59.0/60.1 |
| Curb Weight (lbs., MT/AT) | 2588/2652 | 2659/2720 | 2712/2782 |
| Weight Distribution (%, front/rear, MT) | 60 / 40 | 60 / 40 | 60 / 40 |
| Weight Distribution (%, front/rear, AT) | 61 / 39 | 61 / 39 | 61 / 39 |
| Interior Measurements |  |  |  |
|  | DX | LX | EX/EX-L |
| Headroom (in., front/rear) | 38.0 / 35.1 | 38.0 / 35.1 | 37.8 / 34.7 |
| Legroom (in., front/rear) | 42.6 / 30.3 | 42.6 / 30.3 | 42.6 / 30.3 |
| Shoulder Room (in., front/rear) | 53.9 / 52.1 | 53.9 / 52.1 | 53.9 / 52.1 |
| Hiproom (in., front/rear) | 53.0 / 49.2 | 53.0 / 49.2 | 53.0 / 49.2 |
| Cargo Volume (cu. ft.) | 11.5 | 11.5 | 11.5 |
| Passenger Volume (cu. ft.) | 83.7 | 83.7 | 83.0 |
| Seating Capacity | 5 | 5 | 5 |
| EPA Mileage Estimates**/Capacities |  |  |  |
|  | DX | LX | EX/EX-L |
| 5-Speed Manual (City/Highway/Combined) | 26 / 34 / 29 | 26 / 34 / 29 | 26 / 34 / 29 |
| 5-Speed Automatic (City/Highway/Combined) | 25 / 36 / 29 | 25 / 36 / 29 | 25 / 36 / 29 |
| Crankcase (qt.) | 4.7 | 4.7 | 4.7 |
| Coolant System (qt., MT/AT) | 6.3 | 6.3 | 6.3 |
| Fuel (gal.) | 13.2 | 13.2 | 13.2 |
| Required Fuel | Regular Unleaded | Regular Unleaded | Regular Unleaded |
| Safety |  |  |  |
|  | DX | LX | EX/EX-L |
| 3-Point Seat Belts at all Seating Positions | Standard | Standard | Standard |
| Front 3-Point Seat Belts with Automatic Tensioning System | Standard | Standard | Standard |
| Driver's and Front Passenger's Seat Belt Reminder | Standard | Standard | Standard |
| Dual-Stage, Dual-Threshold Front Airbags (SRS) | Standard | Standard | Standard |
| Front Side Airbags with Passenger-Side Occupant Position Detection System (OPDS) | Standard | Standard | Standard |
| Side Curtain Airbags | Standard | Standard | Standard |
| Driver's and Front Passenger's Active Head Restraints | Standard | Standard | Standard |
| Advanced Compatibility Engineering™ (ACE™) Body Structure | Standard | Standard | Standard |
| Vehicle Stability Assist™ (VSA®) with Traction Control |  |  | EX-L |
| Brake Assist |  |  | EX-L |
| Anti-Lock Braking System (ABS) | Standard | Standard | Standard |
| Electronic Brake Distribution (EBD) | Standard | Standard | Standard |
| Tire Pressure Monitoring System (TPMS)†† | Standard | Standard | Standard |
| Daytime Running Lights (DRL) | Standard | Standard | Standard |
| Side-Impact Door Beams | Standard | Standard | Standard |
| Outboard Lower Anchors and Tethers for CHildren (LATCH) | Standard | Standard | Standard |
| Immobilizer Theft-Deterrent System | Standard | Standard | Standard |
| Emergency Trunk Release | Standard | Standard | Standard |
| Exterior Features |  |  |  |
|  | DX | LX | EX/EX-L |
| One-Touch Power Moonroof with Tilt Feature |  |  | Standard |
| Integrated-Rear Window Antenna | Standard | Standard | Standard |
| Security System with Remote Entry |  | Standard | with Trunk Release |
| Manually Operated Side Mirrors | Standard |  |  |
| Body-Colored Power Side Mirrors |  | Standard | Heated (EX-L) |
| 2-Speed/Intermittent Windshield Wipers | Standard | Standard |  |
| Variable Intermittent Windshield Wipers |  |  | Standard |
| Impact-Absorbing Body-Colored Bumpers | Standard | Standard | Standard |
| Multi-Reflector Halogen Headlights | Standard | Standard | Standard |
| Body-Colored Rear Decklid Spoiler | Standard | Standard | Standard |
| Chrome Exhaust Finisher |  |  | Standard |
| Body-Colored Door Handles |  | Standard | Standard |
| Comfort & Convenience |  |  |  |
|  | DX | LX | EX/EX-L |
| Air Conditioning with Air-Filtration System |  | Standard | Standard |
| Honda Satellite-Linked Navigation System™ with Voice Recognition*** |  |  | Available |
| Bluetooth® HandsFreeLink®††† |  |  | with Navigation |
| Power Windows | Standard |  |  |
| Power Windows with Auto-Up/Down Driver's Window |  | Standard | Standard |
| Power Door Locks with Auto-Lock Feature |  | Standard | Standard |
| Cruise Control |  | Standard | Standard |
| Perforated Leather-Wrapped Steering Wheel |  |  | EX-L |
| Steering Wheel-Mounted Controls |  | Cruise | Cruise/Audio/Navi(available) |
| Tilt and Telescopic Steering Column | Standard | Standard | Standard |
| Beverage Holders | Front | Front | Front |
| Center Console with Sliding Armrest and Storage Compartment |  | Standard | Standard |
| Door-Pocket Storage Bins, Front | Standard | Standard | Standard |
| Passenger-Side Seatback Pocket |  |  | Standard |
| Satin-Finish Door-Handle Pulls |  | Standard | Standard |
| Coin Tray | Standard | Standard | Standard |
| Remote Fuel Filler Door Release | Standard | Standard | Standard |
| Remote Trunk Release with Lock | Standard | Standard | Standard |
| Headlights-On Reminder | Standard | Standard | Standard |
| Driver's and Front Passenger's Vanity Mirrors | Standard | Standard | Standard |
| Ambient Console Lighting |  |  | Blue |
| Map Lights |  | Standard | Standard |
| Floor Mats |  | Standard | Standard |
| 12-Volt Power Outlet | Front | Front | Front and Center Console |
| Driver's Footrest | Standard | Standard | Standard |
| Rear Window Defroster with Timer | Standard | Standard | Standard |
| Rear Seat Garment Hooks | Standard | Standard | Standard |
| Cargo Area Light | Standard | Standard | Standard |
| Seating |  |  |  |
|  | DX | LX | EX/EX-L |
| Leather-Trimmed Seats |  |  | EX-L |
| Heated Front Seats |  |  | EX-L |
| Driver's Seat with Manual Height Adjustment | Standard | Standard | Standard |
| Reclining Front Seatbacks | Standard | Standard | Standard |
| Adjustable Head Restraints | Standard | Standard | Standard |
| Fold-Down Rear Seatback | Standard | Standard | 60/40 Split |
| Passenger-Side Walk-In Feature |  | Standard | Standard |
| Audio Systems |  |  |  |
|  | DX | LX | EX/EX-L |
| 160-Watt AM/FM/CD Audio System with 6 Speakers |  | Standard |  |
| 350-Watt AM/FM/CD Premium Audio System with 7 Speakers including Subwoofer |  |  | Standard |
| MP3/Windows Media® Audio (WMA) Playback Capability**** |  | Standard | Standard |
| MP3/Auxiliary Input Jack |  | Standard | Standard |
| USB Audio Interface†††† |  |  | Standard |
| XM® Radio***** |  |  | with Navigation |
| Welcome Screen/CD Text Display Capability |  | Standard | Standard |
| Digital Audio-Card Reader |  |  | with Navigation |
| Speed-Sensitive Volume Control (SVC) |  | Standard | Standard |
| Instrumentation |  |  |  |
|  | DX | LX | EX/EX-L |
| Two-Tier Instrument Panel | Standard | Standard | Standard |
| Backlit Gauges | Blue | Blue | Blue |
| Tachometer | Standard | Standard | Standard |
| Digital Odometer and Digital Trip Meters (2) | Standard | Standard | Standard |
| Tire Pressure Monitoring System (TPMS) Indicator†† | Standard | Standard | Standard |
| Maintenance Minder™ System | Standard | Standard | Standard |
| Exterior Temperature Indicator |  |  | Standard |
| Fuel and Coolant Temperature Indicators | Standard | Standard | Standard |
| Door/Trunk-Open Indicators | Standard | Standard | Standard |
| Low-Fuel and Low-Oil Pressure Indicators | Standard | Standard | Standard |
| Passenger-Side Side Airbag-Off Indicator | Standard | Standard | Standard |
| Front Passenger Airbag-Off Indicator | Standard | Standard | Standard |

* ULEV-2 rated by California Air Resources Board (CARB).

† Does not apply to fluid and filter changes. Will vary with driving conditions. See your owner's manual for more details.

** Based on 2009 EPA mileage estimates, reflecting new EPA fuel economy methods beginning with 2008 models. Use for comparison purposes only. Do not compare to models before 2008. Your actual mileage will vary depending on how you drive and maintain your vehicle.

†† For optimal tire wear and performance, tire pressure should be checked regularly with a gauge. Do not rely solely on the monitor system. See your local Honda dealer for details.

*** The Honda Satellite-Linked Navigation System™ is available on Civic Coupe EX and EX-L models in the United States, except Alaska. See your Honda dealer for details.

††† The Bluetooth® word mark and logos are owned by the Bluetooth SIG, Inc., and any use of such marks by Honda Motor Co., Ltd., is under license.

**** Windows Media® is a trademark or registered trademark of Microsoft® Corporation in the United States and/or other countries.

†††† The USB Audio Interface is used for direct connection to and control of some current digital audio players and other USB devices that contain MP3, WMA® or AAC music files. Some USB devices with security software and digital rights-protected files may not work. Please see your local Honda dealer for details on compatibility. Windows Media® Audio is a trademark or registered trademark of Microsoft® Corporation in the United States and/or other countries.

***** Required XM® Radio monthly subscription sold separately after 90-day trial period. Installation costs and other fees and taxes may apply after initial trial period, including a one-time activation fee. All fees and programming subject to change. Subscriptions subject to Customer Agreement available at xmradio.com. XM service only available in the 48 contiguous United States. The XM name is a registered trademark of XM Satellite Radio Inc.